COMPUTER NETWORKS

Assignment 1 Report

G SAI SHASANK

22CS10025

Part 1: Networking Tools

1 Task 1: Find the IP Address, Subnet Mask, an Network ID

Using the ifconfig command, we identified the following:

- IP Address of the Machine: 10.5.16.169

- Subnet Mask: 255.255.255.0

- Network ID: 10.5.16.255

Output of ifconfig showing IP address, subnet mask, and network ID.

2 Task 2: Resolve Domain Names using nslookup

Using nslookup, the IP addresses for www.google.com and www.facebook.com were obtained:

- IP Address of www.google.com: 142.250.182.196

- IP Address of www.facebook.com: 31.13.79.35

When the DNS server address was changed to 172.16.1.164, 172.16.1.180, 172.16.1.165, and 172.16.1.166, the IP address of www.google.com was observed to change. This behavior is due to the load-balancing mechanisms used by DNS servers. These mechanisms ensure that traffic is distributed efficiently across multiple servers, improving performance, reliability, and redundancy.

IP address associated with www.google.com and www.facebook.com

IP address of www.google.com with DNS address 172.16.1.164

IP address of www.google.com with DNS address 172.16.1.180

IP address of www.google.com with DNS address 172.16.1.165

IP address of www.google.com with DNS address 172.16.1.166

| DNS | IP Adress |
| --- | --- |
| 172.16.1.164 | 142.250.76.164 |
| 172.16.1.180 | 142.250.207.228 |
| 172.16.1.165 | 142.251.42.4 |
| 172.16.1.166 | 142.250.182.196 |

3 Task 3: Ping Command with Different Packet Sizes

Using the ping command, packets were sent to a friend’s machine with varying sizes:

- Friend’s IP Address: 10.5.16.152

- Packet Sizes: 64, 128, 512 bytes.

- Timeout: 100ms.

Results Table:

The results of the ping command with varying packet sizes are summarized below:

| Packet Size (bytes) | Packet Loss (%) | Round-Trip Time (min/avg/max/stddev) |
| --- | --- | --- |
| 64 | 0 | 2.375 / 8.033 / 28.504/ 5.954 |
| 128 | 0 | 3.398 / 8.363 / 20.583/ 4.853 |
| 512 | 0 | 2.270 / 7.643/ 18.256 / 3.963 |

Screenshots:

The following images show the ping command outputs for each packet size:

Ping output for 64-byte packets.

Ping output for 128-byte packets.

Ping output for 512-byte packets.

4 Task 4: Traceroute Command

Using the traceroute command, the path to www.google.com was analyzed. The number of hosts involved in the path from source to destination was counted.

Summary of Results:

• Number of Hosts: 13

• Observed “* * *” in intermediate hops: These indicate timeouts or unreachable hosts, often caused by network policies or firewalls or TTL expiration without response

Screenshot of the output:

Output of traceroute for www.google.com.

Part 2: Packet Analysis

1. DNS Packets Analysis

- DNS Query/Response Protocol: UDP

- Source IP Address of DNS Query: 10.5.16.169

- Destination IP Address of DNS Query: 172.16.3.10

- Number of DNS Queries Sent: 12

- DNS Server Replying with IP Address: 172.16.3.10

- Number of DNS Servers Involved: 1

Resource Records:

DNS Response						nslookup response

DNS query and response captured in wireshark

2. Web Traffic (HTTP) Analysis

- Number of HTTP Packets Exchanged: 10

- HTTP Request: GET / ̃grovesd/ HTTP/1.1

- HTTP Response: HTTP/1.1 200 OK (text.html)

- HTTP Request

- HTTP Response

- HTTP Request 3.

Figure 14: HTTP Request and Response for http://web.simmons.edu/ grovesd/.

3. ICMP Traffic (Ping/Traceroute)

- Ping to Friend’s IP: 10.5.16.169

- Traceroute to Friend’s IP: 10.5.16.169

- Ping to Unreachable Host: 192.168.1.100

(a) ping and traceroute to 10.5.16.169

(b) ”ping” and ”traceroute” captured in Wireshark.

( c ) Analysis of ping request

- ping to unreachable host(19.168.1.100)

( e ) Trace to unreachable host(19.168.1.100)

(f) ”ping” and ”traceroute” captured in Wireshark

( g ) Analysis of ping request ( unreachable host )